## Managing Firewalld on Fedora Server

Firewalld is the default dynamic firewall management tool for Fedora, providing a simplified way to manage network access rules based on security zones. This document outlines essential commands for configuration and management.

1. Status and Management of Firewalldx

Ensure the firewalld service is running and enabled on system startup.

| Command | Description |
| --- | --- |
| sudo systemctl status firewalld | Checks the current running status of the firewalld service. |
| sudo systemctl start firewalld | Starts the firewalld service if it is inactive. |
| sudo systemctl enable firewalld | Ensures firewalld starts automatically on boot. |

2. Zone Management

Firewalld uses zones to group network interfaces and firewall rules, simplifying management for different network trust levels (e.g., public, trusted, home).

| Command | Description |
| --- | --- |
| sudo firewall-cmd --get-zones | Lists all available predefined zones. |
| sudo firewall-cmd --get-default-zone | Shows the currently active default zone (usually public). |
| sudo firewall-cmd --set-default-zone=your-zone --permanent | Changes the system-wide default zone permanently. |
| sudo firewall-cmd --get-active-zones | Lists zones that currently have active interfaces or sources assigned. |

3. Adding and Removing Rules (Services & Ports)

Rules must be added with the --permanent flag to persist across reboots, followed by a --reload to activate them immediately.

A. Managing Services (e.g., HTTP, HTTPS, SSH)

| Service Command (Public Zone) | Description |
| --- | --- |
| sudo firewall-cmd --zone=public --add-service=http --permanent | Opens port 80 (HTTP) permanently. |
| sudo firewall-cmd --zone=public --add-service=https --permanent | Opens port 443 (HTTPS) permanently. |
| sudo firewall-cmd --zone=public --remove-service=http --permanent | Closes port 80 (HTTP) permanently. |

B. Managing Custom Ports

| Port Command (Public Zone) | Description |
| --- | --- |
| sudo firewall-cmd --zone=public --add-port=8080/tcp --permanent | Opens TCP port 8080 permanently. |
| sudo firewall-cmd --zone=public --add-port=53/udp --permanent | Opens UDP port 53 permanently. |
| sudo firewall-cmd --zone=public --remove-port=8080/tcp --permanent | Closes TCP port 8080 permanently. |

4. Applying and Listing Rules

Changes made with --permanent are not effective until the firewall is reloaded.

| Command | Description |
| --- | --- |
| sudo firewall-cmd --reload | MANDATORY: Loads the permanent configuration changes into the runtime. |
| sudo firewall-cmd --zone=public --list-all | Lists all permanent and active rules (services, ports, sources, masquerading) for the public zone. |
| sudo firewall-cmd --list-all-zones | Lists all permanent rules for all zones. |

5. Advanced Configuration

Masquerading (NAT)

Enabling masquerading is necessary if the server acts as a gateway or router to translate internal IP addresses to the public IP address.

| Command | Description |
| --- | --- |
| sudo firewall-cmd --zone=public --add-masquerade --permanent | Enables IP masquerading in the public zone. |

Rich Rules

Rich rules offer more complex, granular control over packet filtering, allowing specification of source/destination IP addresses, protocols, and actions (accept, reject, drop, log).

| Rich Rule Command | Description |
| --- | --- |
| sudo firewall-cmd --add-rich-rule='rule family="ipv4" source address="192.168.1.0/24" service name="ssh" accept' --permanent | Permanently allows SSH access only from the 192.168.1.0/24 subnet. |
| sudo firewall-cmd --remove-rich-rule='...' --permanent | Removes the specified rich rule permanently. |

Important Security Note: When configuring firewalld, ensure you do not inadvertently lock yourself out of the server (especially SSH) by testing changes on a non-production system or maintaining console/physical access.