## Setting up a Basic VPN with OpenVPN on Fedora

This guide details the setup of a basic OpenVPN server on Fedora (version 40+) for secure, encrypted remote access. OpenVPN is a widely used open-source Virtual Private Network solution.

1. Installation and Directory Setup

Begin by installing the openvpn package and setting up the necessary configuration directory structure.

| Command | Description |
| --- | --- |
| sudo dnf install openvpn easy-rsa | Installs the OpenVPN server package and the easy-rsa tool for managing certificates. |
| sudo mkdir -p /etc/openvpn/server | Creates the directory to hold the server configuration and keys. |
| sudo cp /usr/share/doc/openvpn/server.conf /etc/openvpn/server/server.conf.sample | Copies the sample configuration file. |

2. Certificate Authority (CA) and Key Generation

OpenVPN requires a Public Key Infrastructure (PKI) for secure connections. We use the included Easy-RSA toolset.

2.1. Prepare Easy-RSA

Copy the Easy-RSA scripts to the OpenVPN server directory to manage keys locally.

sudo cp -r /usr/share/easy-rsa/ /etc/openvpn/server/easy-rsa

cd /etc/openvpn/server/easy-rsa

sudo ./easyrsa init-pki

2.2. Build the CA

Create the Certificate Authority (CA), which will sign all server and client certificates.

# Create the CA (answer 'yes' to confirm)

sudo ./easyrsa build-ca nopass

2.3. Generate Server Key and Certificate

Create the server request, sign it with the newly created CA, and generate the Diffie-Hellman parameters.

| Command | Description |
| --- | --- |
| sudo ./easyrsa gen-req server nopass | Generates the server key and request file. |
| sudo ./easyrsa sign-req server server | Signs the server request using the CA. Answer yes when prompted. |
| sudo ./easyrsa gen-dh | Generates the Diffie-Hellman parameters (this may take several minutes). |

2.4. Gather Keys

Copy all generated files to the main OpenVPN server directory.

sudo cp pki/ca.crt pki/dh.pem pki/issued/server.crt pki/private/server.key /etc/openvpn/server/

3. Configure the OpenVPN Server

Edit the main configuration file, /etc/openvpn/server/server.conf.sample, and rename it to server.conf.

sudo cp /etc/openvpn/server/server.conf.sample /etc/openvpn/server/server.conf

sudo nano /etc/openvpn/server/server.conf

Ensure the following lines are present and correctly configured (adjusting paths and IP ranges as needed):

port 1194

proto udp

dev tun

ca ca.crt

cert server.crt

key server.key

dh dh.pem

server 10.8.0.0 255.255.255.0  # VPN subnet

push "redirect-gateway def1 bypass-dhcp"

push "dhcp-option DNS 8.8.8.8"

push "dhcp-option DNS 8.8.4.4"

user nobody

group nobody

persist-key

persist-tun

status /var/log/openvpn/openvpn-status.log

verb 3

explicit-exit-notify 1

4. Network Configuration

4.1. Enable IP Forwarding

The server must be able to forward packets between the VPN interface and the public internet interface.

sudo nano /etc/sysctl.conf

# Uncomment or add the following line:

net.ipv4.ip_forward = 1

# Apply the change immediately

sudo sysctl -p

4.2. Firewalld Configuration (NAT and Port)

Use firewalld to open the OpenVPN port (1194/UDP) and enable IP masquerading (NAT) so VPN clients can access external networks.

| Command | Description |
| --- | --- |
| sudo firewall-cmd --zone=public --add-port=1194/udp --permanent | Opens the default OpenVPN port permanently. |
| sudo firewall-cmd --zone=public --add-masquerade --permanent | Enables NAT/Masquerading for traffic leaving the public interface. |
| sudo firewall-cmd --reload | Reloads firewalld to apply the permanent rules immediately. |

5. Start and Enable OpenVPN

Start the service using the @server instance name, which corresponds to the /etc/openvpn/server/server.conf file.

| Command | Description |
| --- | --- |
| sudo systemctl start openvpn@server | Starts the OpenVPN server instance. |
| sudo systemctl enable openvpn@server | Configures the service to start automatically at boot. |

6. Client Configuration Generation

Repeat steps to generate unique keys for each client device using Easy-RSA. Replace clientname with a unique identifier (e.g., user-laptop).

cd /etc/openvpn/server/easy-rsa

# 1. Generate client request

sudo ./easyrsa gen-req clientname nopass

# 2. Sign client request with the CA

sudo ./easyrsa sign-req client clientname

Transfer the following three files securely to the client device:

- ca.crt (from /etc/openvpn/server/)

- clientname.crt (from easy-rsa/pki/issued/)

- clientname.key (from easy-rsa/pki/private/)