## Troubleshooting Common Networking Issues

Diagnosing network issues efficiently requires a systematic approach, starting with basic connectivity checks and moving toward configuration and service checks.

3.1. Connectivity and Status Checks

| Command | Diagnostic Purpose |
| --- | --- |
| nmcli or ip a | Check Interface Status: Verifies if network interfaces are active, up, and have correct IP addresses assigned. |
| ping -c 4 8.8.8.8 | Test Internet Reachability: Checks outbound connectivity to a reliable external host (e.g., Google DNS). |
| ip route | Verify Routing Table: Confirms that the server has a correct default gateway route configured. |

3.2. Configuration and Service Review

| Area | Action |
| --- | --- |
| DNS Resolution | Check /etc/resolv.conf to ensure it contains valid, working DNS server addresses. Use dig or nslookup to test specific domain resolutions. |
| Firewall Rules | Use sudo firewall-cmd --list-all to ensure the required services (e.g., SSH, HTTP, OpenVPN) are permanently allowed in the correct zone. |
| SELinux Context | If a service is failing or blocked, check SELinux logs (sudo journalctl -t setroubleshoot) and temporarily set SELinux to permissive mode for testing: sudo setenforce 0. (Remember to use semanage for permanent fixes). |
| Configuration Files | Review network configuration files, such as those under /etc/sysconfig/network-scripts/ifcfg-*, for manual errors. |

3.3. Service and Hardware Issues

| Issue Type | Action |
| --- | --- |
| Service Failure | Restart core network services: sudo systemctl restart NetworkManager or sudo systemctl restart network (if not using NetworkManager). |
| IP Conflict | Ensure that no other device on the network is using the server's static IP address. |
| Hardware | Check physical connections (cables, switch ports). For virtual servers, confirm host-level network connectivity is stable. |
| Log Files | Examine system logs (/var/log/messages, /var/log/dmesg) for recent error messages related to network interfaces or service failures. |